# Context

You are the network engineer of the company “skills”. Some users told you that sometime the network is not working; you have 45 minutes to go through the entire network and see what the 5 mistakes are.

# Briefing

This activity will be placed during the day 2, for a maximal total time of 45minutes. 
You can stop this activity whenever you want.

You must not go back to the Windows test project until you give back this activity!

This activity will be assessed anonymously, you MUST NOT add an element to distinguished your work (name, country, station number, … ).

For security reasons, you should work with a local copy of this activity on you workstation.

When finished, only ONE activity must exist on the USB drive!

# Network configuration

The network is divided in 2 site, the first and most important one, is the Headquarter and is divided into 4 VLANs and the second site is a very simple remote site.

For all equipment, there is a common configuration:

| Console access | Yes with password CorpoPasswd |
| --- | --- |
| SSH Access | Yes with user admin and password Passw0rd |

## Headquarter Site

Routing in the Headquarter site is managed by the L3 switch SW1. Remote connection to the branch is managed by R1. SW1 have a static route to the branch site through R1.

### IP Address scheme

| Hostname(s) | Interface | IP |
| --- | --- | --- |
| PC0, PC1, PC2, PC3 |  | DHCP |
| SW1 | Vlan10 | 192.168.10.1 |
| SW1 | Vlan20 | 192.168.20.1 |
| SW1 | Vlan99 | 192.168.99.1 |
| SW1 | Vlan100 | 192.168.100.1 |
| SW2 | Vlan99 | 192.168.99.2 |
| SW3 | Vlan99 | 192.168.99.3 |
| SW4 | Vlan99 | 192.168.99.4 |
| SW5 | Vlan99 | 192.168.99.5 |
| SW6 | Vlan99 | 192.168.99.6 |
| SW7 | Vlan99 | 192.168.99.7 |
| NETSRV |  | 192.168.100.5 |
| SRV |  | 192.168.100.6 |
| R1 | Gi0/0 | 192.168.100.254 |
| R1 | Se0/0/0.201 Frame relay DLCI 201 | 10.0.0.1/30 |

### VLAN

| ID | Name | Network | Clients |
| --- | --- | --- | --- |
| 10 | IT | 192.168.10.0/24 | PC0, PC1 |
| 20 | Direction | 192.168.20.0/24 | PC2, PC3 |
| 99 | Mgmt | 192.168.99.0/24 | All switches |
| 100 | Datacenter | 192.168.100.0/24 | NETSRV, SRV |

### VTP

VTP is configured and working for all switches in the network. VTP server is SW1. VTP domain is skills.org and password is : Passw0rd

### STP

Spanning portfast is enabled by default on all access interfaces. SW1 is the root switch for the network.

### Trunking management

For performance and security reasons, trunks from SW1 must allow only needed VLAN.

It means in Fa0/1 and Fa0/2, only Vlan10,20 and 99 must be allowed and for interfaces Gi0/1 and Gi0/2, only Vlan 99 and 100 are allowed.

### Aggregation

Links between SW1 and SW6 are aggregated with LACP protocol

Link between SW6 and SW7 are aggregated with PaGP protocol.

### Servers

NETSRV is the DHCP server for VLAN 10 and 20.

NETSRV is the DNS server for all clients, it have records for all equipment, and for NETSRV and SRV.

SRV is a Web and email server for all clients (HQ and branch).

Email accounts are the same than PC’s hostnames.

## Branch site

Branch site is a small site connected to headquarter through Frame Relay ISP.

FrameRelay cloud is managed by an external company, you just have access to routers.

Here are configurations given by the provider :

| Hostname | Interface | DLCI | IP |
| --- | --- | --- | --- |
| R1 | Se0/0/0.201 | 201 | 10.0.0.1 / 30 |
| R2 | Se0/0/0.102 | 102 | 10.0.0.2 / 30 |

### Routing

The default route for R1 and R2 is SW1.

### SRVBR

SRVBR is the DHCP server for PC4, it have the IP 10.0.1.5 / 24