# Test Project
Module F - Pre-delivery Inspections

49 Heavy Vehicle Technology

## Submitted by:

## Name: Toromont Cat (Maritimes) Gerry Brymer, Independent Designer

# Contents

This Module (Module F: Pre-delivery Inspections) includes the following documents:

Task 1 Checklist – Perform Pre-delivery Inspection (supplied)

Task 2 Defect List – Repair Pre-delivery Defects (supplied after completion of Task 1)

Oil Sampling Procedures (“How to Take a Good Oil Sample”) (supplied)

Pre-delivery (Revisit) Procedures (supplied)

Volvo’s Product Service Information System (PROSIS) (online)

# Introduction to Test Project

This Test Project consists of six Modules.

You will complete the entire Test project in 18 hours over four days.

You will complete two Modules each day.

You will have three hours to complete each module.

# Description of project and tasks

This Module (Pre-Delivery Inspections) tests for excellence in diagnosing and repairing unsatisfactory conditions and defects.

You will be working on a Volvo articulated wheel loader.

There are two tasks to complete in three hours.

You must complete these tasks in the order given.

For manufacturer’s service information, you will have the supplied documents Oil Sampling Procedures (“How to Take a Good Oil Sample”) and Pre-delivery (Revisit) Procedures, and access to Volvo’s Product Service Information System (PROSIS) (online).

If there is no manufacturer’s service information for diagnosing or repairing an unsatisfactory condition or defect, you must use your own knowledge and procedures.

Your judge can operate the machine for you for testing purposes.

Please be very specific about what you would like the judge to do.

The Marking Scheme is on the last two pages of this document.

# Instructions to the Competitor

## Safety Procedures

### What to wear and not wear

You MUST wear:

Safety glasses with side protection.

Safety-toed boots.

Work clothes.

Mechanic’s gloves for handling hot, sharp, slippery, or heavy components.

Barrier gloves (supplied) for handling fluids.

Long hair tied up or tied back and tucked down the back of the shirt, so none is dangling.

You must NOT wear:

Competitor ID badge. Place it on the toolbox during this competition segment.

Jewellery (rings, bracelets, watches, necklaces, pins).

Neckties.

Lanyards.

Any dangling or loose item that might get caught in a piece of moving equipment.

### How to Avoid Hazards

Before you start, you must go through the following Hazard Assessment with your judge.

You must be able to work safely.

You will not lose marks for asking your judge about this Hazard Assessment.

You must fully understand it before you begin to work.

You will lose marks if you fail to eliminate or control the hazards as you work.

## Hazard Assessment

### The work you will be doing presents 8 hazards.

Here is a list of the hazards and how to eliminate or control them.

# Hazard 1: 	You could slip off the machine while climbing into the operator compartment.

Face the vehicle and position your hands and feet appropriately.

Use three points of contact at all times.

Hazard 2: 	When the engine is operating, the moving parts could injure you.

Stay clear of moving parts.

Hazard 3:	Hydraulic systems could crush you.

Stay clear and keep others clear of pinch zones and implement linkages.

Hazard 4:	Hydraulic components might fail to hold raised equipment and could 				fall on you.

Use the supplied safety supports. The judge will assist you with these safety 				supports.

Lower all implements after testing.

Hazard 5:	Highly pressurized hydraulic oil could pierce your skin.

Ensure that the system is off and depressurized, unless performing 					operational tests.

When performing tests, do not direct pressurized oil toward your body.

Check for leaks using a piece of cardboard or similar material.

Hazard 6:	Track tensioners have stored energy even with the key in the off position.

Before performing service procedures, follow manufacturer’s instructions to relieve trapped pressures and lock out the machine.

Hazard 7:	You could slip and fall on spilled hydraulic oil.

Wipe and clean all oil drips and spills immediately.

Hazard 8:	Hot components and fluids could severely burn you.

Use caution when touching hydraulic and engine parts.

# Tasks

Complete the following two tasks in the order presented.

## Task 1

### Perform Pre-delivery Inspection.

To safely and properly perform this task, follow Volvo’s Product Service Information System (PROSIS) (online) and the supplied documents Oil Sampling Procedures (“How to Take a Good Oil Sample”) Pre-delivery (Revisit) Procedures. If there is no manufacturer’s service information available, use your own knowledge and procedures.

- Inspect the machine in the order given, using the Task 1 Checklist – Perform Pre-Delivery Inspection (supplied).

- Test the machine to confirm that it is ready to be delivered to the customer.

- In point form in the space provided on the Checklist, summarize the unsatisfactory conditions and defects found during inspection.

## Task 2

### Repair Pre-Delivery Defects.

Follow Volvo’s Product Service Information System (PROSIS) (online) to safely and properly perform this task. If there is no manufacturer’s service information available, use your own knowledge and procedures.

From your judge, obtain the Task 2 Defect List – Repair Pre-Delivery Defects.

- Make necessary repairs to reduce the list.

If a specific part is required to perform the repairs, ask your judge if it is available.

- Test the machine to confirm that it is ready to be delivered to the customer.

- In point form in the space provided on the Defect List, summarize what you did to return the system to its normal operation.

# Equipment, machinery, installations, and materials required

No requirements to post.

# Marking Scheme

### From the WorldSkills Standards Specification (WSSS) for Heavy Vehicle Maintenance

Each Module is marked as follows:

Safety – 10.4%

Consistently and diligently follow the best procedures to protect health and safety in the working environment

Use appropriate personal protective equipment (safety footwear and eye protection with side shields at all times, and ear protection, respiratory protection, and either barrier gloves or fitted mechanic’s gloves, as needed)

Select and handle substances, materials, and equipment safely and in compliance with manufacturers’ instructions

Dispose of substances and materials safely and sustainably

Predict and eliminate all risks related to required activities

Prepare and maintain an orderly workspace with regard to health and safety, and restore the workspace for the next technician

Logical Order of Repair – 12.8%

Organize and implement appropriate decisions regarding maintenance or repair

Use the methods best suited to complete each task

Use and Interpretation of Technical Information – 12.8%

Choose the appropriate sources of technical information applicable to the task

Read, interpret, and extract technical information from the chosen sources

Apply technical information to the task

Understand and accurately use the technical language associated with the task

Precision Measurement – 12.8%

Demonstrate an understanding of the types of diagnostic and precision measurement tools in both imperial and metric units

Demonstrate an understanding of the purposes, proper handling, and use of the types of diagnostic and precision measurement tools

Choose, use, and interpret the results of diagnostic and precision measurement tools to produce accurate measurements to determine component reusability and to find faults in components and systems

Fault-Finding – 12.8%

Recognize and diagnose faults in heavy vehicle components or systems

Choose, use, and interpret the results of appropriate diagnostic methods and equipment

Apply the results of diagnostic testing and any relevant calculations to correctly identify and isolate faults related to the task

Appropriate Use of Tools – 12.8%

Choose and properly use, maintain, and store appropriate tools for the task

Maintenance or Repair of Components or System – 12.8%

Choose the appropriate procedures to meet manufacturers’ specifications for maintenance or repair of diesel engine systems, hydraulic systems, pneumatic systems, electrical and electronic systems, drive train systems, and pre-delivery inspection

Predict and alleviate the effects of the chosen procedures on other components or systems

Communication of Maintenance or Repair Process – 12.8%

Clearly and accurately record technical information in a written report about each task