# Test Project

Mechatronics

# Project A1:

## Assembly, programming and commissioning of a Handling station with electrical drive and stack magazine together with a Joining station

| t max | - Information |
| --- | --- |
| 180 min | - also on USB-Stick |

### Scenario

| You are responsible for the delivery of a Handling Station with a Joining Station purchased by a customer in the world to be used in the partial automation of its production process. |  |
| --- | --- |

### Task

Assemble, wire and tube the Handling station on the profile plate according to the following guidelines and the technical information.

Develop a program and commission the system.

Your task is complete when:

The production line has been mechanically assembled, correctly wired, connected and its correct operation is guaranteed (based on evaluation using the simulation box).

Correct execution of the program with PLC activation (based on evaluation with PLC) is guaranteed.

The system meets the specifications (in accordance with the 'Agreement on Professional Practice' Judgment which has been handed out separately already before the Competition).

Scenario: The system will be sent to the customer as soon as you are finished. Hardware problems during the evaluation phase can be solved afterwards in the following task.

© WorldSkills International (WSI) reserves all rights in documents developed for or on behalf of WSI, including translation and electronic distribution. This material may be reproduced for non-commercial vocational and educational purposes provided that the WorldSkills International logo and copyright notice are left in place.

### Mechanical information - Production line layout:

Place your HMI here

1

2

Screws to turn the axis

Distance block

Sensor plate

5

4

3

- Handling station (HS)

Magazine pickup position

Slide drop down position 1

Slide drop down position 2

table position

### Mechanical information - Optical Sensor at Magazine pick up position:

- Change the light barrier in the module Stack magazine to the reflection sensor in the magazine pick-up position.

### Mechanical information - Back side of the Handling unit

Positions of:

Multipol I/O module

Motor controller

Valve terminal

Fixing of the linear axis

Fixing of the linear axis

Optical Sensor for Gripper

### Electrical information - Wiring allocation of sensors to Multipol (MPV)

|  | Pin | Core colour | M8 socket / Pin | Signal IN | function | take your note here for the I/O terminal |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 0 / 4 | 0 | Gripper is up |  |
|  | 2 | Brown | 1 / 4 | 1 | Gripper is down |  |
|  | 3 | Green | 2 / 4 | 2 | Gripper unit is in table position (left) |  |
|  | 4 | Yellow | 3 / 4 | 3 | Gripper unit is in magazine position |  |
|  | 5 | Grey | 4 / 4 | 4 | Gripper unit is in Joining station position |  |
|  | 6 | Pink | 5 / 4 | 5 | workpiece in Gripper is not black |  |
|  | 7 | Blue | 6 / 4 | 6 | Not used |  |
|  | 8 | Red | 7 / 4 | 7 | Not used |  |
|  | 9-12 | – | – | – | – |  |
|  | 13 | White-green | 0-7 / 1 | 24V DC |  |  |
|  | 14 | Brown-green | 0-7 / 3 | 0V |  |  |
|  | 15 | White-yellow | 0-7 / 3 | 0V |  |  |

### Electrical information - Wiring allocation valve terminal

|  | PIN | Core colour | Coil | Signal Out | Function | take your note here for the I/O terminal |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 0 | 0 | Move gripper down |  |
|  | 2 | Brown | 1 | 1 | Open gripper |  |
|  | 3 | Green | 2 |  | Not used |  |
|  | 4 | Yellow | 3 | 2 | Not used |  |
|  | 5-13 | – | – | – |  |  |
|  | 14 | Brown-green |  | 0V |  |  |
|  | 15 | White-yellow |  | 0V |  |  |

### Electrical information - Motor controller R/L:

A1: move to right side / A2 : move to left side (see from the front side/Gripper unit)

1, 2: Motor

VCC: 24 V / GND: 0V

A3 and I-OUT are not used

| Name at Motor Controller | take your note here for the I/O terminal |
| --- | --- |
| VCC | 24V |
| GND / 0V | 0V |
| I-OUT | Not used |
| A1 | DO3 |
| A2 | DO4 |
| A3 | Not used |

### Task on Joining Station (JS)

- Adjust the sensors and mechanical components for a proper process if necessary

- Use the signals via your distributed device of your Handling Station for the communication with the joining station

Words:

Parts = Workpiece

### Communication Joining station with Handling station

| Description (HS) | Bit |  | Bit | Description (JS) |
| --- | --- | --- | --- | --- |
| Part at Conveyor left hand side. | I0 |  | Q4=1 | Part at Conveyor left hand side |
| Part at Conveyor right hand side | I1 |  | Q5=1 | Part at Conveyor right hand side |
| Station not busy | I2 |  | Q6=1 | Station not busy |
| Not used | I3 |  | Q7 | Not used |
| Process 1 (in / back) | Q0=1 |  | I4 | Process 1 (in / back) |
| Process 2 (in / to the end) | Q1=1 |  | I5 | Process 2 (in / to the end) |
| Start of process (Strobe) | Q2=1 |  | I6 | Start of process (Strobe) |
| Start of Conveyor belt 1 (350 mm) | Q3=1 |  | I7 | Start of Conveyor belt 1 (350 mm) |

- For each Workpiece you place in the Joining Station you have to set the Start Signal

- The Conveyor 1 in the JS stops by detection of the optical sensors.

### Communication Joining station with Handling station in Task A

The Joining Station (downstream station) is sending a not busy signal via the Mini I/O-Terminal as soon as the station is in initial position. This signal can be detected via the Mini I/O-Terminal at the Handling station (upstream station) in order to achieve a safe material flow.

By switching on or off this signal, the Joining Station signals that HS can transport a workpiece or JS is busy.

### Timing diagramm of the Joining station (JS)

CB2

CB1

### Attention:

- Keep the cables from the top of the profile plate to the I/O Terminal long. Don’t cut the cable shorter. It’s allowed that the cables hang in the trolley but they have to bundle with cable ties

- If there is no cap on the conveyor CB2 for more than 1 min. the CB2 will stop and you have to reset the station and start again.

### Details of Module Stack Magazine (SM) view (more on USB –Stick)

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Ejecting arm extended |
|  | 2 | Brown | 7 | Q0 |  | Ejecting arm push out workpiece |
|  | 3 | Green | 2 | I1 |  | Ejecting arm retracted |
|  | 4 | Yellow | 8 | Q1 |  |  |
|  | 5 | Gray | 3 | I2 |  | Workpiece is in magazine pickup position |
|  | 6 | Pink | 9 | Q2 |  |  |
|  | 7 | Blue | 4 | I3 |  |  |
|  | 8 | Red | 10 | Q3 |  |  |
|  | 9 | Black | 5 | AI0 |  |  |
|  | 10 | Violet | 6 | AI1 |  |  |
|  | 11 | Gray-rose | 11+12 | AQ0 |  |  |
|  | 12 | Red-blue | 24VA | VCC-Out | 24 V |  |
|  | 13 | White-green | 24VB | VCC – In | 24 V |  |
|  | 14 | Brown-green | GND A | GND Out | 0 V |  |
|  | 15 | White-yellow | GND B | GND In | 0 V |  |

### Important Information needed for Evaluation

Initial position:

| Handling Station (HS): | Joining Station (JS): |
| --- | --- |
| Stack magazine ejecting arm retraced | Vacuum off |
| Gripper unit in table position | Horizonal slide retracted |
| Gripper closed | Vaccum gripper up |
| Gripper up | Conveyor belt 1 empty |
| Magazine and Magazine pick up position empty (not relevant during the process but for the RESET!) | Conveyor belt 2 (cap transportation) minimum one cap available |
| Table empty |  |

| Field 1 | Field 2 | Field 3 | Field 4 |
| --- | --- | --- | --- |
| Field 5 | Field 6 | Field 7 | Field 8 |
| Field 9 | Field 10 | Field 11 | Field 12 |
| Field 13 | Field 14 | Field 15 | Field 16 |

Use the HMI (your own HMI) with the following applications

Wiring on the board (see also Specification)

T2

T1

To the interface in Joining Station

To the I/O interface of your distributed I/O Device

To the I/O interface of your PLC

Space for Distr. device

### According to the colour and the selection the work piece will be transported to different positions.

|  | wrong Workpiece | red with cap | Black | Silver with cap |
| --- | --- | --- | --- | --- |
| Selection | different | X | X | X |
| Process after identification |  |  |  |  |
| On Slide 1 |  | X |  | X |
| On Slide 2 |  |  | X |  |
| On the table on the left-hand side | X |  |  |  |

### Attention:

Keep the cables from the top of the profile plate to the I/O Terminal long. Don’t cut the cable shorter. It’s allowed that the cables hang in the trolley, but they have to bundle with cable ties

### Note:

HMI – Field 6: to show the Words like in the design or like in the specification, both is correct for the customer.

Wording: Workpiece = WP = Piece