# Test Project

Mechatronics

# Project B:

## Assembly, programming and commissioning of a production line including Handling station, 3-Magazin station and Joining station

| t max | Information |
| --- | --- |
| 330 min | also on USB-Stick |

### Scenario

| You are responsible for the delivery of a Handling Station with a Distribution Station that delivers three different workpieces purchased by a customer in the world to be used in the partial automation of its production process. The product is a pot with different colours and delivered with a cap |  |
| --- | --- |

### Task

Assemble, wire and tube the production line on the profile plate according to this documentation and commission the system.

Your task is complete when:

The production line has been mechanically assembled, correctly wired, connected and its correct operation is guaranteed (based on evaluation using the simulation box).

Correct execution of the program with PLC activation (based on evaluation with PLC) is guaranteed.

The system meets the specifications (in accordance with the 'Agreement on Professional Practice' judgment).

The system will be sent to the customer as soon as you are finished. You will have no opportunity to make improvements later. Hardware problems during the evaluation phase can be solved during the next task.

© WorldSkills International (WSI) reserves all rights in documents developed for or on behalf of WSI, including translation and electronic distribution. This material may be reproduced for non-commercial vocational and educational purposes provided that the WorldSkills International logo and copyright notice are left in place.

### Production line layout

3

2

1

5

6

4

| - Handling station (HS) | - Joining Station (JS) |
| --- | --- |
| - 3-Magzine station (3MS) | - Module Conveyor 700 mm |
| - Module Magazine | - Module Identification |

### Details of 3 Magazine station (3MS)

### System layout

Center line

Move away

3

2

1

Groove 14

Groove 1

### Assembly of the Motor at the Conveyor System

Note: The distance between adapter plate and coupling is 2 mm! Coupling is already assembled but not the motor on the conveyor!

### Assembly of IO-Link Device, Flow Meter and Signal Tower

Note:

- C-Rail: Groove 3 / left-hand side of the station

- Flowmeter: on the left-hand side on the c-rail

- IO-Link Device: on the right-hand side on the c-rail

- Signal lamp: Groove 5 / right-hand side of the station between IO-Link Device and Magazin

- Identification Modul between Magazine 1 and 2 or left of Magazine 1 (Decision of the competitors)

- All cables from all Modules to the I/O-Terminal is to keep in the maximal length.

- Mark all cables from the Magazine to the IO-Link device on both ends with cable ties (1 cable tie port A, 2 cable ties port B)

You can find the IODD on the USB-Stick!

Please connect up to 2 of the three Magazines on the IO-Link DA Interface.

Note:

In the optimal setup you prepare your Station that it is wired with I/O-Signals via the Terminals 1 and 2 and also via IO-Link. You decide how the Magazines are connected during the PLC evaluation.

SimBox evaluation will be done via the cable that goes from the Mini IO- Terminal to the Terminal 1 or/and 2

### Add the flow sensor to the 3MS to measure the total flow of all stations

### Energy measurement for air consumption

| Connect the 24V/0V and analogue output to the analogue I/O-Terminal at the wiring board. Measure the air flow for the whole system. Don’t cut the cable short (keep it as ring and hand let it hang in the trolley) | Select RECORDER mode Start / Stop and RESET the measurement with the sensor buttons |
| --- | --- |

### RECORDER mode

In the RECORDER mode, a manual accumulated air consumption measurement can be performed.

- Press the A and B button at the same time. The SFAB changes into the RECORDER mode. The status of the air consumption measurement [Run] or [Stop] is displayed.

- If [Stop] is displayed, press the A button to change to [Run]. [Run] and a running light is displayed. The air consumption measurement is started.

- Press the A button again to change back to [Stop]. [Stop] is displayed. The air consumption measurement is stopped.

Note:

Resetting measurement value to zero.

- Press the B button in the RECORDER mode to reset a measurement value to zero.

Communication between Handling Station, Joining Station and 3-Magazin Station in Task B

### Communication between Joining station and PLC in Handling station via Mini i/O Terminal on the distributed device

See Task A

### Different real views see annex (more on USB –Stick)

### Details of Modules see annex (more on USB –Sick)

### Details of Module Conveyor 700 x 40 in the 3MS Station

### Details of Module Stackmagazin in the 3MS Station

### Details of Module identification

## Annex

## Different real views

| Top view | Wiring to the terminal board in the trolley (example) | Module Identification on the conveyor |  |
| --- | --- | --- | --- |

| Separator | Optical sensor tower | Reflex-Light Fibre optic |
| --- | --- | --- |

| Stopper back | Stopper front | Light barrier fibre optic |
| --- | --- | --- |

## Modul information

### Details of Module Conveyor 700 x 40 in the 3MS Station (more on USB –Stick)

### Electrical information - Wiring allocation Mini-Terminal

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal Fill in by yourself | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Workpiece on conveyor start / left |
|  | 2 | Brown | 7 | Q0 |  | Conveyor belt moves in direction to the Handling Station |
|  | 3 | Green | 2 | I1 |  | Workpiece at stopper |
|  | 4 | Yellow | 8 | Q1 |  | Conveyor belt moves in direction to the left-hand side |
|  | 5 | Grey | 3 | I2 |  | Workpiece at conveyor end position / right |
|  | 6 | Pink | 9 | Q2 |  | retract stopper / separate WP |
|  | 7 | Blue | 4 | I3 |  | Not used |
|  | 8 | Red | 10 | Q3 |  | Not used |
|  | 9 | Black | 5 | AI0 |  | Not used |
|  | 10 | violett | 6 | AI1 |  | Not used |
|  | 11 | Gray-rose | 11+12 | AQ0 |  | Not used |
|  | 12 | Red-blue | 24VA | VCC-Out |  |  |
|  | 13 | White-green | 24VB | VCC – In |  |  |
|  | 14 | Brown-green | GND A | GND Out |  |  |
|  | 15 | Whith-yellow | GND B | GND In |  |  |

### Details of Module Stackmagazin in the 3MS Station (more on USB –Stick)

### Electrical information - Wiring allocation Mini-Terminal

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal Fill in by yourself | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Ejector retracted |
|  | 2 | Brown | 7 | Q0 |  | extend Ejector |
|  | 3 | Green | 2 | I1 |  | Ejector extended |
|  | 4 | Yellow | 8 | Q1 |  | Not used |
|  | 5 | Grey | 3 | I2 |  | Magazine empty |
|  | 6 | Pink | 9 | Q2 |  | Not used |
|  | 7 | Blue | 4 | I3 |  | Not used |
|  | 8 | Red | 10 | Q3 |  | Not used |
|  | 9 | Black | 5 | AI0 |  | Not used |
|  | 10 | violett | 6 | AI1 |  | Not used |
|  | 11 | Gray-rose | 11+12 | AQ0 |  | Not used |
|  | 12 | Red-blue | 24VA | VCC-Out |  |  |
|  | 13 | White-green | 24VB | VCC – In |  |  |
|  | 14 | Brown-green | GND A | GND Out |  |  |
|  | 15 | Whith-yellow | GND B | GND In |  |  |

### Details of Module identification

### Electrical information - Wiring allocation Mini-Terminal

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal Fill in by yourself | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Inductive Sensor |
|  | 2 | Brown | 7 | Q0 |  | Not used |
|  | 3 | Green | 2 | I1 |  | Optical sensor: Light barrier |
|  | 4 | Yellow | 8 | Q1 |  | Not used |
|  | 5 | Grey | 3 | I2 |  | Optical sensor: Reflex light |
|  | 6 | Pink | 9 | Q2 |  | Not used |
|  | 7 | Blue | 4 | I3 |  | Optical sensor: Height sensor digital |
|  | 8 | Red | 10 | Q3 |  | Not used |
|  | 9 | Black | 5 | AI0 |  | Optical sensor: Height sensor analogue |
|  | 10 | violett | 6 | AI1 |  | Not used |
|  | 11 | Gray-rose | 11+12 | AQ0 |  | Not used |
|  | 12 | Red-blue | 24VA | VCC-Out |  |  |
|  | 13 | White-green | 24VB | VCC - In |  |  |
|  | 14 | Brown-green | GND A | GND Out |  |  |
|  | 15 | Whith-yellow | GND B | GND In |  |  |

### Important Information

Initial position:

| Handling Station (HS): | Joining Station (JS): |
| --- | --- |
| See Task A: | See Task A: |

3-Magazine Station (3MS):

Module conveyor motor off

Module conveyor stopper extended

Module conveyor empty (not relevant during the process but for the RESET!)

Stack magazine ejecting arm retraced

Magazines empty (not relevant during the process but for the RESET!)

### Details of touchpanel Design for Evaluation

| Field 1 | Field 2 | Field 3 | Field 4 |
| --- | --- | --- | --- |
| Field 5 | Field 6 | Field 7 | Field 8 |
| Field 9 | Field 10 | Field 11 | Field 12 |
| Field 13 | Field 14 | Field 15 | Field 16 |

### Definition of Stack Magazines and storage color!

|  | red | Black | Silver | Transparent |
| --- | --- | --- | --- | --- |
| Magazine in Handling Station | X |  |  |  |
| Magazine in 3MS left-hand side (1) |  | X |  |  |
| Magazine in 3MS middle (2) |  |  |  | X |
| Magazine in 3MS right-hand side (3) |  |  | X |  |

### According to the colour and the selection the work piece will be transported to different positions.

|  | upside down | red | Black | Silver | Transparent |
| --- | --- | --- | --- | --- | --- |
| Selection |  | X | X | X | X |
| Process after identification |  |  |  |  |  |
| On Slide 1 | X |  |  |  |  |
| On Slide 2 |  | X | X | X | X |
| With cap |  | X | X | X |  |

### Attention:

- Conveyor: You have a pneumatic Stopper and an electrical Separator in the box. It’s your decision which component you like to use to have a problem free function

- Conveyor: You have 2 x light barrier fibre optics and 3 x light reflex fibre optics. It’s your decision which type of fibre sensor you use on which of the three locations along the conveyor.

Only 3 have to be used!

- Magazines: You have for each Magazine one light barrier fibre optics. Additional you get a light reflex fibre optics. It’s your decision how to use the additional component.

- All the not used components have to be stored in the provided box at the end of the Task.