# Test Project

Mechatronics

# Project C:

## Assembly, programming and commissioning of a production line including Handling station, 3 Magazin station, Vision Quality station and Joining station

| t max | Information |
| --- | --- |
| 330 min | also on USB-Stick |

### Scenario

| You are responsible for the delivery of a Handling Station with a 3 Magazine Station, Vision Quality Station and Joining Station purchased by a customer in the world to be used in the partial automation of its production process. |  |
| --- | --- |

### Task

Assemble, wire and tube the production line on the profile plate according to this documentation.

Your task is complete when:

The production line has been mechanically assembled, correctly wired, connected and its correct operation is guaranteed (based on evaluation using the simulation box).

Correct execution of the program with PLC activation (based on evaluation with PLC) is guaranteed.

The system meets the specifications (in accordance with the 'Agreement on Professional Practice' judgment).

The system will be sent to the customer as soon as you are finished. You will have no opportunity to make improvements later. Hardware problems during the evaluation phase can be solved during the next task.

© WorldSkills International (WSI) reserves all rights in documents developed for or on behalf of WSI, including translation and electronic distribution. This material may be reproduced for non-commercial vocational and educational purposes provided that the WorldSkills International logo and copyright notice are left in place.

### Production line layout

5

2

4

1

3

| - Handling station (HS) | - Joining Station (JS) |
| --- | --- |
| - 3 Magazine station (3MS) | - Vision Quality Station (VQS) |
| - Modul Camera | - Modul Stopper-Ejector |

2

3

1

6

### Details of Vision Quality station (VQS)

### Communication between Handling, Joining, 3-Magazin Station and Vision Quality Station in Task C

### Communication between Joining station and PLC in Station Handling station via Mini i/O Terminal on the distributed device

See Task A

### different real views see annex (more on USB –Stick)

### Details of Modules see annex (more on USB –Stick)

### Details of Module Conveyor 350 x 40 in the Vision Quality  Station

### Details of Module Stopper-Ejector

## Annex

## Different views

### Different views (VQS)

| D: Camera | E: Optical sensor tower | F: Stopper |
| --- | --- | --- |

| G: Lighting | H: Camera box | Assembly of windows |
| --- | --- | --- |

| Optical mirror | System layout Top view Station | Important: silicon washer under the frame of the moving window |
| --- | --- | --- |

| Stopper-Ejector | Note: Assembly alignment | Assembly alignment |
| --- | --- | --- |

## Modul information

### Details of Module Conveyor 350 x 40 in the Vision Quality Station (more on USB –Stick)

### Electrical information - Wiring allocation Mini-Terminal

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal Fill in by yourself | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Workpiece on conveyor start |
|  | 2 | Brown | 7 | Q0 |  | Conveyor belt moves in direction to the slide |
|  | 3 | Green | 2 | I1 |  | Workpiece at stopper |
|  | 4 | Yellow | 8 | Q1 |  | Conveyor belt moves in direction to the camera |
|  | 5 | Grey | 3 | I2 |  | No workpiece at conveyor end position / slide |
|  | 6 | Pink | 9 | Q2 |  | retract Stopper |
|  | 7 | Blue | 4 | I3 |  | Not used |
|  | 8 | Red | 10 | Q3 |  | Not used |
|  | 9 | Black | 5 | AI0 |  | Not used |
|  | 10 | violett | 6 | AI1 |  | Not used |
|  | 11 | Gray-rose | 11+12 | AQ0 |  | Not used |
|  | 12 | Red-blue | 24VA | VCC-Out |  |  |
|  | 13 | White-green | 24VB | VCC – In |  |  |
|  | 14 | Brown-green | GND A | GND Out |  |  |
|  | 15 | Whith-yellow | GND B | GND In |  |  |

### Details of Module Stopper-Ejector in the Vision Quality station (actuators and sensors)

### Electrical information - Wiring allocation Mini-Terminal

|  | PIN on SUB-D | Colour DIN47100 | Pin I/O-Mini-Terminal | Pin-Function | Pin I/O-Terminal Fill in by yourself | Function |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | White | 1 | I0 |  | Stop-Cylinder retracted |
|  | 2 | Brown | 7 | Q0 |  | extend Stop-Cylinder |
|  | 3 | Green | 2 | I1 |  | Ejector-Cylinder retracted |
|  | 4 | Yellow | 8 | Q1 |  | extend Eject-Cylinder |
|  | 5 | Grey | 3 | I2 |  | Workpiece at stop-cylinder |
|  | 6 | Pink | 9 | Q2 |  | Not used |
|  | 7 | Blue | 4 | I3 |  | Not used |
|  | 8 | Red | 10 | Q3 |  | Not used |
|  | 9 | Black | 5 | AI0 |  | Not used |
|  | 10 | violett | 6 | AI1 |  | Not used |
|  | 11 | Gray-rose | 11+12 | AQ0 |  | Not used |
|  | 12 | Red-blue | 24VA | VCC-Out |  |  |
|  | 13 | White-green | 24VB | VCC – In |  |  |
|  | 14 | Brown-green | GND A | GND Out |  |  |
|  | 15 | Whith-yellow | GND B | GND In |  |  |

### Important Information

Initial position:

| Handling Station (HS): | Joining Station (JS): | 3-Magazine Station (3MS): |
| --- | --- | --- |
| See Task A: | See Task A: | See Task B: |

Vision Quality Station (VQS):

Module conveyor: Motor off

Module conveyor: Stopper extended

Module conveyor: Belt empty

Module Stopper-Ejector: retracted

### Details of touchpanel Design for Evaluation

### Use the HMI (your own HMI) with the following applications

| Field 1 | Field 2 | Field 3 | Field 4 |
| --- | --- | --- | --- |
| Field 5 | Field 6 | Field 7 | Field 8 |
| Field 9 | Field 10 | Field 11 | Field 12 |
| Field 13 | Field 14 | Field 15 | Field 16 |

### According to the colour and the selection the work piece will be transported to different positions.

|  | without inlay |  | upside down | red |  | Black |  | Silver |  | Trans-parent |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T |  |  | RFID | wo RFID | RFID | wo RFID | RFID | wo RFID | With Inlay |
| Selection on HMI | X |  |  | X |  | X |  | X |  |  |
| Process after identifycation |  |  |  |  |  |  |  |  |  |  |
| On Slide in VQS |  |  | X |  | X |  | X |  | X |  |
| On Slide 1 | X |  |  |  |  |  |  |  |  |  |
| On Slide 2 (and all wrong colours) |  | X |  |  |  |  |  |  |  | X |
| With cap on conveyor End in JS |  |  |  | X |  | X |  | X |  |  |

Note:

If the system detects a wrong workpiece then selected the WP will go out of the System like written in the specification and a new one has to be distributed from the Magazine.

Colours in the magazines are the same like in Task B

### Example for the Workpieces

Inlay with RFID- Chip

==>

Note: If the Workpiec will be upside down there will not be an inlay in the WP!

### Vision Quality detection – Sensortype: Object SBSI-Q-R3B-F6-W

Note: Don’t set a Password for the Camera system!!

Sensor for Online mode

Start the Software:

- Choice: Simulation Mode Sensor for Simulation mode: Object, R3B, Standards

- Choice: Online with the Camera / Active sensor – Find mode

Sensor for Simulation mode

### Steps to do – Simulation Mode!

- Select Sensor for Simulation Mode ==> Vision Sensor Configuration Studio - Object.

- Job definition:

Tip: Orientation

Tip: RFID Chip

- Alignment ==> Method not needed

- Detector ==> You can check your application/Jobs offline by using the filmstrip on the USB!

Orientation: Tip: Contrast, Brightness or Gray

RFID Chip: Tip: Contrast, Brightness or Gray with 2 x Detectors true as standard

- Output ==> Standard ==> Pin definition like below

- Result ==> Method not needed

### Steps to do! – Online Mode

- Search for Sensor, modify your IP-range ==> Double click: User admin, no Password.

- Job definition:

Tip: Orientation

Tip: RFID Chip

- Alignment ==> Method not needed

- Detector ==> You can check your application/Jobs offline by using the filmstrip on the USB!

Orientation: Tip: Contrast, Brightness or Gray

RFID Chip: Tip: Contrast, Brightness or Gray with 2 x Detectors true as standard

- Output ==> Standard ==> Pin definition like below

- Result ==> Method not needed

- Start Sensor

Trigger for Image test/update

Stop Sensor for modifications

### I/O mapping to set – Pin deffinition – Online

### PIN assignment, connection 24 V DC

| PIN | Color | Use | I/O Terminal | Setting |
| --- | --- | --- | --- | --- |
| 1 | BN | + Ub (24V DC) | 24 V | 24 V |
| 2 | BU | GND | 0 V | 0 V |
| 3 | WH | IN (external trigger) | DO 0 | Trigger |
| 4 | GN | READY *1 | DI 1 | Ready |
| 5*2 | PK | IN/OUT |  |  |
| 6*2 | YE | IN/OUT |  |  |
| 7*2 | BK | IN/OUT, LED B*4 |  |  |
| 8*2 | GY | IN/OUT, LED C*4 | DI 2 | Job change confirm/ready |
| 9 | RD | OUT (external illumination) |  | External Illumination (camera internal) |
| 10 | VT | IN | DO 1 | Job change 1 or 2 |
| 11 | GYPK | VALID *3 | DI 0 | Result Valid |
| 12 | RDBU | OUT (ejector, max. 100mA), LED A*4 | DI 3 | Result 0=NOK; 1=OK |

*1 Ready: Ready for next ext. trigger Trigger.

*2 Switchable input- output

*3 VALID: shows available results

*4 All LED´s are set without taking into account any timing function (e.g. Trigger delay)

Take image

Change job

Take image

### binary switching and Job 1 or 2:

By usage of binary job switch the desired job number must be represented binary coded via the selected inputs. In case of Job change Job 1 or 2 a high level cause’s evaluation according to job 2, a low level produces evaluation according to job 1. In this way with only one input the switching between two jobs can be done.