# Test Project

Mechatronics

# Project E:

## optimization in a production line

| t max | Information |
| --- | --- |
| 60 min | also on USB-Stick |

### Scenario

| The production line needs to be optimized and you have to plan and test this optimization to reduce the production time and to find the most energy efficient production process. |  |
| --- | --- |

### Task

Your task is to measure the airflow, to optimize the production process to produce an order in the shortest possible time and with most air efficiency to bring out the right order.

Your task is complete when:

- Correct execution of the program with PLC activation (based on evaluation/PLC board) is guaranteed.

- The system meets the specifications (in accordance with the 'Agreement on Professional Practice / Judgment').

You will have no opportunity to make improvements later.

© WorldSkills International (WSI) reserves all rights in documents developed for or on behalf of WSI, including translation and electronic distribution. This material may be reproduced for non-commercial vocational and educational purposes provided that the WorldSkills International logo and copyright notice are left in place.

### Procedure of optimization:

Safety: Don’t work at the stations while the stations are running!

Target:

Fast, energy efficient and dependable production line

Conditions:

It is allowed to modify your PLC – programs

The focus on energy consumption is on air

Work pieces from the magazine in the Handling station and 3MS will be processed

Max. 6 bar working pressure / Max. 24 Volt power supply

No collisions are allowed (Workpieces touching each other side by side is not a collision)

During your commissioning, evaluation and testing you can stop the calculation of the flow-sensor via the stop function in the RECORDER mode. See Task B.

It is allowed to adjust /move the sensors and modules in the same groove within the station, but the function must remain the same.

It is not allowed to use tape in the production line.

Exceptions:

More than 1 workpiece on the production system is allowed

Press start only one time (workpieces are running automatically)

Open the End block at the Conveyor end of the Joining Station. Workpieces can be falling out.

The workpieces are moving without manual help.

No evaluation about behaviour of the lamps except start lamp at beginning of evaluation and signal lamps at the end.

The timing and air flow measurement stops when the GREEN, RED and YELLOW signal lamps are ON in all stations and the order is completed.

After a max. of 60 Minutes optimization the system has to be prepared to run:

There are no points for the preparation time

All modules and components are fixed and adjusted.

The station layout must be the same as in the tasks before.

The Stations are ready to get the Start signal via the Start button at the HMI; therefore, the Start Lamp must be ON at the HMI.

After the optimization time 3 members of the evaluation team will take the production time and note the air consumption displayed on the Sensor. The average of these 3 times will be calculated for each