# Plumbing and Heating

# Test Project

WSC2015_TP15_actual

Submitted by:

Name: Paul Dodds

Member country/region: United Kingdom

## Introduction to Test Project documentation

## Contents

This Test Project proposal consists of the following documentation/files:

WSC2015_TP15_UK_EN.docx

WSC2015_TP15_UK_EN order form.docx

WSC2015_TP12_BR_02_EN.pdf 	 A2 Drawing Scale 1:20

A3 Details Drawing (Not to Scale)

## Introduction

The Test Project for Worldskills Sao Paulo 2015 presents the installation typical systems installed in a domestic dwelling on a pre- fabricated structure, the installation includes:-

Ordering Materials for the task

Module 1 Sanitation System Installation of Sanitary Appliances

Module 2 Hot and Cold Water Systems

Module 2 and 4 Gas System

Heating System including Under Floor Heating and Towel rail

Live Installation of sanitary appliance as a repair/replacement including commissioning.

The project developed will be assessed using integrated assessment which will include objective criterion and the introduction of the new assessment methodology of Judgement, there will be no subjective marking.

## Description of project and tasks

The Test Project consists of a number of tasks that need to be completed within a 21hr timescale

The order for completion is as follows:-

- Design of towel rail to given parameters and the ordering of the material for the Towel Rail

- Ordering of materials for completion of Module 2 &3  -

- Installation of Towel Rail (module 3)

- Installation of Module 3 to be handed over end of Day 2

- Installation of Module 2 to be handed over end of Day 3

- Installation of Module 1 to be handed over to customer end of Day 4

- Installation of Module 4 to be handed over end of Day 4

- During working time of Day 1,2, and 3 a live installation to be completed – Max 2hr (Possible to return to workstation after 1hr if Live task complete)

Instructions to the Competitor
The Test Project consists of a number of tasks that need to be completed within a 21hr timescale with deadlines for the handover of work in stages for Assessment.
The order for completion is as follows:

- Design of towel rail to given parameters and the ordering of the material for the Towel Rail (Max 1hr at this point material order to be handed in. If additional time is required see note below in relation to Task A and B))
You are required to design a towel rail using 15mm copper pipe to given design requirements of the customer they are:
Given the design parameter of 15mm pipe emitting ???? Watts/per metre and that the towel rail in total needs to emit???Watts. (The data ???? will be drawn prior to start of Module 4)
The design needs to be in 2D or 3D Competitor choice and of a QUALITY that it could be constructed/interpreted by an external contractor if required. The Competitor notes/text on this drawing the Competitor can use his own native language. The E or I is requested after the Task time to translate it into English.
The drawing/sketch has to include all dimensions and sufficient information for the contractor. Include your calculation for the pipe required for the towel rail, which must be made from one continuous length of pipe.

You need to ensure that your design is capable of holding a towel on a straight 600mm horizontal pipe as a minimum. 
The completed Towel Rail will Judged by the general Public so be as creative as possible, bends may be of any angle to suit Competitors design.
The design parameters/drawing/sketch and calculation for pipe will be assessed using objective marking.

- Ordering of materials for completion of Module 2 and 3 - (Task A and B Total 2hrs, no installation work will start until after this time)
If additional time is required then the Competitor must use some of their competition time for the task. It should however be note that the handover times for modules must still be met or incur penalty for not complete in time.

- Delivery of materials for Towel Rail completed by WM

- Installation of Towel Rail (module 3) This must be installed first.

- Delivery of materials for Module 1,20 and 3 completed by WM

- Installation of Module 3 to be handed over end of day 2. The installation of the copper heating pipework (Soft Solder)/Radiator and the PEX underfloor heating NOTE: The installation must be tested for leaks prior to Final hand over (the testing must be witnessed by allocated Experts in the time allocated). The Bend angles that must be used are only “30°, 45°, 60°, 90°” except for underfloor heating which must be as drawing

- Installation of Module 2 to be handed over end of day 3. This includes all the galvanised gas pipe and the PEX hot and cold water installation. Note: The Hot & Cold Water and Gas pipe must be tested for leaks prior to handing over to the customer. (2 bar for 2mins). The Bend angles that must be used are only “30°, 45°, 60°, 90°”.

- Installation of Module 1 to be handed over to customer end of day 4. All sanitary pipework to be fitted and washbasin to be installed as a minimum complete with waste pipe. NOTE: Hot &Cold water connections will be marked day 3.

- Installation of Module 4 to be handed over end of day 4. Also to be completed for day 4 is an extra task where the customer has changed their mind and an alteration needs to be made to the gas supply to suit the customers’ requirements. The Gas supply will need to be changed with the removal of section pipe between two fixed locations (this section of pipe may only be removed from the pipe clips and altered to meet the requirements and also tested (witnessed by allocated Experts) before final handover at the end of the day.

- During working time of Day 1, 2, and 3 a 2hr live installation to be completed. (Note: If the task is completed after 1hr the competitor on completion can return to their workstation)
This installation will consist of the following scenario:
The customer has decided that they want the toilet suite replaced with a new one. 
The task will require the de-commissioning of the existing WC suite and installing a new one. 
The task will require the minimum of tools to be used and ensure that the work is carried out with no damage to the customer’s property, so precautions must be taken. 
On completion of the work the Competitor will commission the installation and check for correct function etc. (flushing single/dual flush, water level) prior to handing when complete. (Demonstration will be given at familiarisation)
The WC Suite taken out will be used in the main project by the Competitor as per drawing.