# Test Project

Plumbing and Heating

Submitted by:

Paul Dodds SCM

# Contents

Introduction to Test Project	3

Introduction	3

Description of project and tasks	3

Instructions to the Competitor	6

Equipment, machinery, installations, and materials required	9

Marking Scheme	9

Other	9

COMPATRIOT COMMUNICATION	9

OUTSIDE COMPETITION TIME	9

# Introduction to Test Project

The Test Project for Kazan 2019 is a major change to previous competitions and contains a lot of Firsts for Plumbing and Heating.

This competition for the first time we will have a 100% Blind Project that has been developed independently.

It is also the first time that Competitors do not bring their own tools to a competition, so all Competitors will have the same equipment to make the project making for a fairer competition and reducing our Carbon Footprint in the transportation of tools across the World.

The first time we will have speed tests that will test the Competitor’s ability to work at a quick pace with the challenge to problem solve and maintain quality.

A WorldSkills first in partnership with our Industry Partner IAPMO (The International Association of Plumbing and Mechanical Officials) to deliver a team task on Day 4 of the competition building modules that will be installed after the competition at Derbyshkinskiy Orphanage in Kazan. This has been a massive team effort and not possible without the help of the Ministry of Building, Architecture and Housing Municipal Services of The Republic of Tatarstan and WorldSkills International.

The University in Kazan, TECE, DHL, 3M and AutoDesk our Sponsors have also supported this project.

# Introduction

This competition relates to a medium sized building and systems associated with this type of building the systems installed will be:

Sanitation System, using push fit plastic pipe

Heating System, using a lightweight Steel pipe and pressed fittings

Hot and Cold-Water Systems, using copper pipe incorporating soldered and pressed fittings

These systems will be developed using a manufactured walling system used in the industry and the installation of appliances associated with the systems.

The competition will also consist of two speed tests, that will test the Competitor’s ability to work at a commercial speed, that test technical pipe skills, problem solving and testing etc. in a pressurised environment.

# Description of project and tasks

Each Competitor will have a personalized timetable for the completion of the tasks and allocated lunch and refreshment breaks. The sequence of Modules will be determined by the timetable and where handover is required this will be included in the Module brief to Competitors.

The projects and tasks below will also identify handover points and if the Module is completed by an Individual or a Team and if it is assessed or not assessed.

Module A 13.5Hrs – Individual Task and Assessed using Measurement and Judgement.

This module is split into several tasks detailed below:

Task 1 – Completed on C1 - Assessed on C2

Manufacture a towel rail with a given length of pipe and to the specification given in the Instructions to the Competitor section. This task is linked to the Sustainability Award for the Skill and the Competitor that uses the pipe most efficiently with high quality will receive a Medallion from our Skill Sponsor IAPMO for Sustainabilty of Natural Resources.

Tested with air on C2

Drawing WSC2019_TP15_SCM_02_EN

If a Competitor requires additional materials carrying out Task 1 marks will be lost.

Task 2 – Completed on C2 and Handed Over – Task 1 and 2 Assessed on C2

To complete the partially assembled wall structure with the appliance base modules fitted into the structure.

Please note that the two Middle level horizontal rails will be the datum for the fabricated wall structure, and these will be adjusted to the required position by the Competitor and Expert. This rail will be positioned on Familiarization Day with the assistance of the compatriot Expert due to safety requirements and the need for an extra pair of hands as would happen in Industry.

Install the heating System and its components (Radiators). This task requires the Competitor to work to the dimensional accuracy, specification, and detail provided in the working drawings provided for this task. Drawing - WSC2019_TP15_SCM_01_EN and WSC2019_TP15_SCM_02_EN

The Competitor must test Task1 and 2 with air as detailed in the Assessment Document

WSC2019_ASD15_EN_V2019.3.1. All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials whilst carrying out Task 2 marks will be lost.

If the Competitor completes the task before the end of C2 and want to move on to Task 3 this is permitted. However, the Competitor cannot carryout any further work on Task 2.

Task 3

Install the Hot/Cold Water system and components, Sanitation Pipework and Sanitary appliances. This will also require the fitting of the wall panels and insulation provided for the task

This task requires the Competitor to work to the dimensional accuracy, specification and detail provided in the working drawings provided for this task. Drawing - WSC2019_TP15_SCM_03_EN

The Competitor must test Task 3 with air as detailed in the Assessment Document (Hot/Cold Water Only) WSC2019_ASD15_EN_V2019.3.1. All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials whilst carrying out Task 3 marks will be lost.

Module B 1.5Hrs – Individual Task and Assessed using Measurement and Judgement.

The scenario for this task is that an engineer was called to the location to replace two faulty pumps. After the Engineer had removed these the Engineer was called to another Job and you have been asked to replace the faulty pumps with new pipework and pumps between two fixed flanges.

The Competitor is to Install new heating pipework, valves and pumps between the fixed flanges as per the drawing provided. Drawing WSC2019_TP15_SCM_04_EN and Assessed as per Assessment Document WSC2019_ASD15_EN_V2019.3.1.

The pipework is 28mm Copper and the jointing used will be by soldering and pressing of fittings as per drawing. The Grundfos pumps must be fitted with the flow of water from Left to Right

The task is to be completed in a maximum time of 1.5hrs, the Task will be tested with water and all air removed from the installation. The Competitor must demonstrate to the Assessment Team that the air is removed, and all valves are open. The Competitor must check for leaks and rectify before handing over.

If a Competitor requires additional materials they should note – NONE WILL BE ISSUED FOR THIS MODULE.

If a Competitor finishes ahead of time, The Competitor MUST remain at the location allocated until the 1.5Hrs is complete and continue to work as per Competitor Timetable.

Module C 1.5Hrs – Individual Task and Assessed using Measurement and Judgement.

This Task tests the Competitors ability to complete a complex pipework Task as per Drawing WSC2019_TP15_SCM_05_EN in a fixed timescale and to the high standards specified in the Assessment Document WSC2019_ASD15_EN_V2019.3.1.

The Competitor is to fabricate the 20mm steel pipe by the use of hydraulic pipe bending and the use of threaded joints. Drawing WSC2019_TP15_SCM_04_EN

The Competitor must test Task 3 with air as detailed in the Assessment Document WSC2019_ASD15_EN_V2019.2.  All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials they should note – NONE WILL BE ISSUED FOR THIS MODULE.

If a Competitor finishes ahead of time, The Competitor MUST remain at the location allocated until the 1.5Hrs is complete and continue to work as per Competitor Timetable.

C4 IAPMO Project Derbyshkinskiy Orphanage – This Project is NOT Assessed

On C4 IAPMO will co-ordinate the activities for the day.

The Competitors will be divided up into groups to work together as a Team in the development of Modules that will be shipped to the home on C+1 and a Team of Experts/Competitors, IAPMO  and students from a Local College will install in the home prior to departure to their home country on 31/8/19. During the Teams will be visited by Reresentatives and Children from the home to experience the work undertaken prior to arrival at the home for the Install. As mentioned earlier this project is something we have tried to do for a number of past competitions and at long last it has become a reality. This is a WorldSkills first and it is hoped we will complete a similar project in Shanghai during WorldSkills 2021.

# Instructions to the Competitor

These instructions may be increased and improved at the Competition during the preparation time and these can be translated for the Competitor by the Expert/ Translator. All Notes to be made on this document.

This Document MUST be handed back at the End of Each Competition Day.

Please ensure that during Familiarisation you have positioned your Horizontal TECE components that will act as your Datum with your Expert giving you assistance. (See Module A - Task 2)

Module A 13.5Hrs – Individual Task and Assessed using Measurement and Judgement.

This module is split into several tasks detailed below:

Task 1 – Completed on C1 - Assessed on C2

Manufacture a towel rail with a given length of pipe and to the specification given in the Instructions to the Competitor section. This task is linked to the Sustainability Award for the Skill and the Competitor that uses the pipe most efficiently with high quality will receive a Medallion from our Skill Sponsor IAPMO for Sustainabilty of Natural Resources.

Tested with air on C2

Drawing WSC2019_TP15_SCM_02_EN

If a Competitor requires additional materials carrying out Task 1 marks will be lost.

The Competitor will be given a length of pipe and required to design a Towel Rail to fit inside the designated aron the wall as seen in the Drawing.

The Towel Rail MUST consist of a minimum

8 x 90o Machine Bends

2 x 45o Machine Bends

2 horizontals pipes uninterrupted for min. 400mm for the hanging of a Towel

The towel must be 3 dimensional and to satisfy this requirement one of the Horzontal Rails must be a minimum 100mm in front of the other horizontal rail

Min 300mm between bend centres for assessment

The pipework will need to be secured to the wall using pipe supports provided.

The Competitor MUST design the Towel Rail to be as efficient as possible in the use of pipe. The competition here is to meet the specification and use the least pipe possible. The Competitor that meets the specification and returns the most pipe will receive maximum marks as detailed in the Assessment Document WSC2019_ASD15_EN_V2019.3.1.

The Towel Rail is to be in one continuous piece of pipe, if additional materials are used then there is a penalty applied for additional material as detailed in the assessment document. For this task only 1 additional fitting will be allowed and a stated penalty will be applied if used.

The Competitor is to make the joints to the valves and position the valves as per drawing in preparation for Task 2. The joints and fittings will be assessed on C2.

All joints must be made in accordance with the Assessment Document and completed before moving on to

Task 2.

All excess material must be handed back to the assessment Team and measured before moving on to Task 2.

Task 2 – Completed on C2 and Handed Over – Task 1 and 2 Assessed on C2

The fittings and 28/22mm Pipe for this Task will be in your competition area for the Module as these were secret items on the IL a list will be provided for checking. The 15mm Pipe will only be provided after your excess material from Task 1 has been measured and recorded by the assessment Team.

To complete the partially assembled wall structure with the appliance base modules fitted into the structure.

The pre-erected wall system has been assembled for you and it is the Competitors responsibility to adapt the structure for the inclusion of all the components requested as per drawing. This will require the cutting of various pieces of steel using the battery powered saw. Please listen carefully on preparation day to the safety advice given when using this equipment.

Please note that the two Middle level horizontal rails will be the datum for the fabricated wall structure, and these will be adjusted to the required position by the Competitor and Expert. This rail will be positioned on Familiarisation Day with the assistance of the compatriot Expert due to safety requirements and the need for an extra pair of hands as would happen in Industry. (no other changes to be made at this time)

Install the heating System and its components (Radiators). This task requires the Competitor to work to the dimensional accuracy, specification and detail provided in the working drawings provided for this task. Drawing - WSC2019_TP15_SCM_01_EN & WSC2019_TP15_SCM_02_EN

The Competitor is to manufacture the pipework as detailed in the drawing taking care to ensure that the Flow and Return connections are the correct way around and connected to the correct valve either on the radiator or towel rail.

The pipework will need to be secured to the wall using pipe supports provided.

The Competitor must test Task1&2 with air as detailed in the Assessment Document WSC2019_ASD15_EN_V2019.3.1. An air line and connectors are provided for this Task and the pressure of the test is specified in the Assessment Document.  All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials whilst carrying out Task 2 marks will be lost.

If the Competitor completes the task before the end of C2 and want to move on to Task 3 this is permitted. However, the Competitor cannot carryout any further work on Task 2.

The Assessment Document details what and how the Task will be assessed.

Task 3

Install the Hot/Cold Water system and components, Sanitation Pipework and Sanitary appliances. This will also require the fitting of the wall panels and insulation provided for the task

The pipework will need to be secured to the wall using pipe supports provided.

The pipework for the Hot/ Cold Water is Copper and will have both Soldered and Pressed Joints. Machine bending of copper is also required. Please ensure that the connections to the appliances and pipework are the correct way around. See the Assessment Document for detailed information. This task requires the Competitor to work to the dimensional accuracy, specification and detail provided in the working drawings provided for this task. Drawing - WSC2019_TP15_SCM_03_EN

The Competitor must test Task 3 with air as detailed in the Assessment Document WSC2019_ASD15_EN_V2019.3.1 (Hot/Cold water Only). All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials whilst carrying out Task 3 marks will be lost.

Module B 1.5Hrs – Individual Task and Assessed using Measurement and Judgement.

The scenario for this task is that an engineer was called to the location to replace two faulty pumps. After the Engineer had removed these the Engineer was called to another Job and you have been asked to replace the faulty pumps with new pipework and pumps between two fixed flanges. Please do not move the flanges when fixing your connection to the flamges already installed as other Competitors will be using the Rig when you have finished.

A Team of Experts will bring the completed work close to your workarea for display to vistors.

The Competitor is to Install new heating pipework, valves and pumps between the fixed flanges as per the drawing provided. Drawing WSC2019_TP15_SCM_04_EN and Assessed as per Assessment Document WSC2019_ASD15_EN_V2019.3.1.

The pipework is 28mm Copper and the jointing used will be by soldering and pressing of fittings as per drawing. The Grundfos pumps must be fitted with the flow of water from Left to Right

The pipework will need to be secured to the wall using pipe supports provided.

The task is to be completed in a maximum time of 1.5hrs, the Task will be tested with water and all air removed from the installation. The Competitor must demonstrate to the Assessment Team that the air is removed, and all valves are open. The Competitor must check for leaks and rectify before handing over. The Competitor can remove the air by the partial undoing of a union until water is present. Please clean up any spilt fluids.

When Soldering remember to use Long Sleeves and PPE identified in the Skill Specific Rules.

If a Competitor requires additional materials they should note – NONE WILL BE ISSUED FOR THIS MODULE.

If a Competitor finishes ahead of time, The Competitor MUST remain at the location allocated until the 1.5Hrs is complete and continue to work as per Competitor Timetable.

Competitors must take their own Tools/Consumables to complete the Module to the assessment location. If a competitor has to return to his/her workstation to collect missing Tools/Consumables, then this is completed in the Module Time.

Module C 1.5Hrs – Individual Task and Assessed using Measurement and Judgement.

This Task tests the Competitors ability to complete a complex pipework Task as per Drawing WSC2019_TP15_SCM_05_EN in a fixed timescale and to the high standards specified in the Assessment Document WSC2019_ASD15_EN_V2019.3.1.

The Competitor is to fabricate the 20mm steel pipe by the use of hydraulic pipe bending and the use of threaded joints. Drawing WSC2019_TP15_SCM_04_EN

Please listen very carefully at Familiarisation on the use of threading and bending equipment and ensure you use the equipment as explained.

All Joints will be assessed as detailed in the Assessment Document. The pipework will need to be secured to the wall using pipe supports provided.

The Competitor must test Task 3 with air as detailed in the Assessment Document WSC2019_ASD15_EN_V2019.2.  All testing must be completed in the competition time and witnessed by the Official team responsible for testing. Competitors can test on their own first and rectify faults prior to asking for the Official test.

If a Competitor requires additional materials they should note – NONE WILL BE ISSUED FOR THIS MODULE.

If a Competitor finishes ahead of time, The Competitor MUST remain at the location allocated until the 1.5Hrs is complete and continue to work as per Competitor Timetable.

Competitors must take their own Tools/Consumables to complete the Module to the assessment location. If a competitor has to return to his/her workstation to collect missing Tools/Consumables, then this is completed in the Module Time.

# Equipment, machinery, installations, and materials required

All Equipment and materials are detailed in the Infrastructure List.

For each Module a list of materials will be provided for checking purposes by the Competitor and faults notified with 15 Minutes of starting Module / Task.

# Marking Scheme

Skill 15 has a well- established Assessment Document WSC2019_ASD15_EN_V2019.2. which gives indepth detail of how work will be assessed with the use of text and photos.

The Competitors will also receive the Mark Summary form for the Assessments prior to the competition.

# Other

Competitors are asked to remember when competing this competition is to simulate a real working environment and you are only to hand over if it is completed in the spirit of the competition demonstrating good practice and that it is how you hand over to a custumer and expect to be paid for the job with all the technical components completed. (See assessment document)

Please note the Following for this Competition for C1, C2, and C3. Rules and Code of Ethics.

### Rule 7.3.3 COMPATRIOT COMMUNICATION AND CONTACT DURING COMPETITION TIME

Competition time is the time spent by the Competitor at their workstation on their Test Project. No contact may be made with the compatriot Expert during competition time without the presence of a non-compatriot Expert. No contact may be made with other Competitors or guests during competition time without the permission of the Chief Expert.

COMPATRIOT COMMUNICATION

(For Skill 15 this will be held with Everybody in the Experts Room)

Periods of 15 to 30 minutes (For Skill15 – the timetabled session is for 15 Minutes) are to be scheduled each morning and evening of the Competition for official communication between compatriot Experts and Competitors (compatriot communication). Communication must be verbal only. The use of any equipment to record or exchange information – such as pens, paper, mobile phones, and electronic devices – is prohibited. (For Skill 15 during communication time these items and Devices including Smart Watches must be kept in the Competitor and Expert Lockers provided and not brought into the Expert Room)

Experts are not permitted to give any help to Competitors to interpret the Test Project unless agreed by the Jury before the start of the skill competition. (For Skill 15 the Experts are NOT to Interpret the task it is important that it is the Competitors own work and knowledge that is assessed and not the skills of the Experts as Two Modules require problem solving to complete the task)

Any questions arising must be referred to the Skill Management Team.

OUTSIDE COMPETITION TIME

Competitors and their compatriot Expert may communicate at any time outside competition time, except skill competitions involving fault-finding. In these cases, Competitors are served lunch in the workshop.

(Experts and Competitors are reminded of the Rules and Code of Ethics in relation to Communication all breaches with evidence will be processed in accordance with the Conflict Resolution Process, The competition works on Trust, Fairness and Transparency and would ask that all Experts and Competitors respect this)

If a Competitor asks for assistance under No Circumstances must more than two Experts enter the Competitors Work area unless part of an assessment Team with an allocated Task.